The Trustees of Reservations, GIS Metadata Form

Dataset Name: Bay_Circuit_Trail

Description and Purpose:

Bay Circuit Trail is a recreational trail open to the public extending through 37 towns in eastern Massachusetts. This data consists of lines representing this trail. This data includes both Dedicated Trail (permanently established routes) and Temporary Trail. This data is periodically subject to updates and edits.

Users of this data take full responsibility for their own safety and actions while hiking the trail. Sections of the trail are along active roads, some of which do not include sidewalks. Use caution and respect private property.

Bay Circuit Trail dataset is intended for personal use only.

Use of this data for other than personal use requires acknowledgement of the Bay Circuit Alliance.

Use of this data for commercial purposes, without the permission of the Bay Circuit Alliance, is prohibited.

For more information on the Bay Circuit Trail and Greenway contact:

The Bay Circuit Alliance

www.baycrcuit.org

Date created:5/10/2010	Date last modified:3/30/2013	Status:

Coordinate System: 					Type:

Mass. State Plane Coordinates

NAD83

Meters

FIPS 2001

Contact Address:

Vincent Antil, GIS Manager

The Trustees of Reservations

464 Abbott Avenue

Leominster, MA 01453

Contact Phone: 978-840-4446 x.1933			Contact E-mail: vantil@ttor.org

Data Source (thematic sources and/or base map):

Almost all lines originated with GPS collection in the field.

See attribute field SOURCE for details.

Detailed Method (if applicable):

Line work was collected in the field using a variety of GPS units.  Primary unit was Garmin Map60, using 3 second reading intervals with pole-mounted GA 27C external antenna. Tracks were edited in Garmin MapSource and ArcGIS to remove major spikes and more closely match features visible on the MassGIS 30cm orthophotos (2008-09).

Attributes (table from ArcGIS):

Trail_Desi                  text  25

Trail_Name                text  50

Other_Name              text  50

Source                       text  75

Update_Dat               date

Notes                         text  50

Shape_Leng              numeric

Leng_Miles                numeric

Field Name - description

Value = description

Trail_Desi - Trail Designation. Describes a section of trail's status.

DEDICATED = Permanently open

TEMPORARY = currently open but not expected to be part of final trail route.

Trail_Name - Name of trail.

Note: This case all trails are named "Bay Circuit Trail".

Other_Name - Name of trail if another trail is coincident with Bay Circuit Trail.

Note: This attribute is not complete. It is primarily used where the Bay Circuit Trail is

following a temporary route or another long distance trail.

Source  -  Source of initial data collection.

Notes  -  Comment field open to any text.

Shape_Leng  -  Computer generated length of line segment in meters.

Leng_Miles  -  Length of line segment in miles.